PMEF

Plant Model Exchange Format

CHANGELOG & RELEASE NOTES

Version 0.9.0-rc — Release Candidate — 2026-03-31

Specification • Schemas • Catalogs • Benchmark Datasets • Rust Implementation

# Release Summary — v0.9.0-rc

First public release candidate of the PMEF Specification. This release covers the complete five-layer architecture (Core Ontology → Information Model → Serialisation → Geometry → Adapters) for five engineering disciplines: Piping, Equipment, E&I, Structural Steel, and Pipe Stress.

## Package Statistics

| Artifact | Count | Notes |
| --- | --- | --- |
| Specification chapters | 6 | 3,087 lines · 18,128 words · normative |
| JSON Schemas | 9 | Draft 2020-12 · all validated |
| Open catalogs | 7 | 733 entries total · CC0 Public Domain |
| Benchmark datasets | 5 | 131 objects · 0 schema errors · CC0 |
| Conformance tests | 82 | 80 passing (97.6%) |
| Rust source lines | 3,723 | 6-crate workspace · Apache 2.0 |
| Total package files | 75 | ~1.2 MB uncompressed · 253 KB ZIP |

# What's New in v0.9.0-rc

## Normative Specification Chapters

Six normative Markdown chapters covering all aspects of the PMEF standard:

- Chapter 01 Introduction — scope, 30+ normative references, 25 term definitions, 45 abbreviations, problem statement (8-column format comparison), 9 design goals, five-layer architecture, P&ID→3D→EAM linking model

- Chapter 02 Information Model — entity type registry (50+ types), all base types with validation, piping domain (13 component subtypes + topology rules), equipment domain (10 subtypes + Nozzle connector), E&I domain, structural steel, 10 relationship types, 11 property sets, extension mechanism

- Chapter 03 Serialisation — NDJSON primary format with canonical serialisation rules, PMEFX container format, CAEX XML secondary format, delta package protocol

- Chapter 04 Geometry — 15 parametric primitive types fully specified, 5 LOD levels, glTF 2.0 + STEP AP242 + OpenUSD layer specs, primitive-to-component mapping table

- Chapter 05 Adapters — 8-step export + 6-step import pipeline, identity resolution algorithm, 8 general requirements, 6 tool-specific specs (E3D, Plant3D, CADMATIC, Tekla, COMOS, EPLAN), full PCF round-trip mapping + unit conversion tables

- Chapter 06 Conformance — 3 conformance levels (L1/L2/L3) with 32 normative clauses, entity type matrix (50+ × 3 levels), 5 benchmark datasets, round-trip fidelity definition, conformance report schema, certification process

## JSON Schemas

9 schemas, all JSON Schema Draft 2020-12, all validated against their own metaschema and the full example dataset:

- pmef-base.schema.json — shared types: PmefId, RevisionMetadata, GeometryReference, CatalogReference, Port, Coordinate3D

- pmef-piping-component.schema.json — PipingNetworkSystem → PipingSegment → 13 component subtypes

- pmef-equipment.schema.json — 10 equipment subtypes + Nozzle

- pmef-geometry.schema.json — 15 parametric primitive types

- pmef-property-sets.schema.json — all 11 property sets

- pmef-ei.schema.json — 6 E&I entity types with full SIL/safety spec

- pmef-steel.schema.json — 4 structural steel entity types

- pmef-relationships.schema.json — 10 typed relationship types

- pmef-catalog.schema.json — catalog file schema (new)

## Open Catalogs (CC0 Public Domain)

| File | Entries | Standard | Description |
| --- | --- | --- | --- |
| profiles-en.json | 202 | EN 10034/10210/10219/10056 | HEA/HEB/IPE/UPE/RHS/SHS/CHS/L/Flat — full section properties |
| profiles-aisc.json | 119 | AISC 16th Ed. (2023) | W / HSS Rect / HSS Round / L-angles — US + metric |
| pipe-dimensions.json | 162 | ASME B36.10M + B36.19M | DN15–DN1200 · 12 schedules · OD/wall/bore/mass |
| piping-class-a1a2.json | 203 | ASME B16.5/B16.9/B16.11 | CS ANSI-150, DN15–DN600 · 9 component types |
| piping-class-b3c1.json | 143 | ASME B16.5/B16.9 | CS ANSI-300, DN15–DN400 · 8 component types |
| caesarII-cii-mapping.json | 29 records | CAESAR II 2024 | PMEF↔CII field mapping + ROHR2 .ntr addendum |
| rdl-uri-map.json | 35 | ISO 15926-4 PCA-RDL | URI cross-references for equipment and piping types |

## Benchmark Datasets (CC0 Public Domain)

| Dataset | Objects | Disciplines | Key Content |
| --- | --- | --- | --- |
| DS-01 pump-skid-complete | 35 | Piping + Equipment + Steel | P-201A (API 610, VFD), V-201 (EN 13445), CW-201/202/203, welds, spool, geometry |
| DS-02 heat-exchanger-station | 26 | Piping + Equipment + E&I | E-301A/B (TEMA BEM, 52 m²), FV-30101 control valve, FIC-30101 HART controller |
| DS-03 compressor-train | 34 | Piping + Equipment + E&I | K-101 (API 617, 2.2 MW), ST-101 (API 611), V-101 KO drum, SIL2 anti-surge |
| DS-04 ei-loop-fic-10101 | 9 | E&I | FIC-10101 SIL1, XV-10101, S7-1500 CPU + AI module, cable, control loop |
| DS-06 stress-model-cw201 | 13 | Piping + Stress | CAESAR II model — anchor reactions, spring hanger, stress ratios (Sus/Exp) |

# Rust Reference Implementation (pmef-rs/)

6-crate Cargo workspace, 3,723 lines, Apache 2.0. Rust 1.75 minimum edition.

## pmef-core — Data Model (1,831 lines)

- PmefId with domain/project/local_id parsing, FromStr validation, underscore support

- PmefVersion with semver compatibility check

- Coordinate3D with distance_to() and JSON array serialisation

- ChangeState enum + RevisionMetadata with factory methods wip() and shared()

- PmefEntityType enum (50 variants) with from_type_str()/as_type_str()

- PmefEntity trait — id, revision, is_part_of, custom_attributes, to_ndjson_line()

- PmefVisitor trait — 45 visit methods with default noop implementations

- PmefAdapter trait — name, version, target_system, conformance_level

- AdapterError (6 variants), AdapterStats, VisitorControl (Continue/Skip/Stop)

- All 50+ entity structs for piping (13 component types via macro), equipment (10 types), E&I (6 types), steel (4 types), relationships (10 types)

- Domain methods: npsh_margin(), lmtd_counter_current(), bore_mm(), effective_wall_mm(), length_mm(), is_spring()

## pmef-io — Streaming NDJSON (375 lines)

- NdjsonReader<R: BufRead> — Iterator over RawPmefObject, skip_comments, fail_fast, 8 MB line limit

- NdjsonWriter<W: Write> — canonical key ordering, write_entity<T: Serialize>

- canonical_json() — deterministic serialisation for checksums and Git diffs

- PmefPackageIndex — in-memory index with resolve() (fragment-stripping) and by_type()

- 4 unit tests: reader, comment skip, canonical JSON, package index

## pmef-cli — Command Line Tool (466 lines)

- 7 subcommands: validate, diff, stats, convert, index, conformance, check-refs

- clap 4 derive API, Tokio async runtime, tracing logging

- validate: JSON Schema validation with conformance level threshold

- diff: object-level diff between two PMEF packages (added/removed/changed)

- check-refs: resolve all @id, isPartOf, sourceId, targetId references

## pmef-adapter-plant3d — PCF Adapter (672 lines)

- PcfUnits: to_mm(), pressure_psi_to_pa_abs(), temp_f_to_k()/temp_c_to_k()

- pcf_keyword_to_pmef_type() + pcf_keyword_to_component_class() — full PCF keyword tables

- MaterialMapper — 15 Plant3D/PCF material strings → PMEF material designations

- parse_pcf() — line-by-line state machine for all PCF keywords + END-POINT blocks

- build_coord_index() — coordinate-based port topology resolution

- Plant3DAdapter::export_pcf_to_pmef() — full export pipeline with HasEquivalentIn

- 9 unit tests covering all major functions

# Schema Fixes (Found by Test Suite)

| Fix | Schema | Details |
| --- | --- | --- |
| PmefId underscore | pmef-base | _ now allowed in local-id segment (required for IDs like FL-WELD_NECK) |
| PIPE_SUPPORT enum gap | pmef-property-sets | PIPE_SUPPORT added to componentClass enum |
| insulationType enum | pmef-equipment | Short forms HOT/COLD used (not THERMAL_HOT/THERMAL_COLD) |
| revision field missing | pmef-geometry | Added to ParametricGeometryObject |
| operatingTemperature location | pmef-piping-component | Removed from PipingSpecification (belongs in designConditions only) |

# Known Issues

| Test ID | Description | Fix Target |
| --- | --- | --- |
| EQUIP-U-005 | KNOCK_OUT_DRUM / ELECTRIC_ARC_FURNACE not in vesselSubtype enum | v0.9.1 |
| PIPE-U-003 | Error message assertion too strict on lineNumber error text | v0.9.1 |
| pmef-geom crate | Full AABB computation for 15 primitives is a stub | v1.0 |
| pmef-validate crate | jsonschema Rust crate integration is a stub | v1.0 |
| DS-05 dataset | Mixed EAF + E&I + Steel benchmark not yet produced | v0.9.1 |
| CADMATIC adapter | REST API adapter (pmef-adapter-cadmatic) not yet produced | v0.9.1 |

# Roadmap

## v0.9.1 — Planned

- Fix EQUIP-U-005 and PIPE-U-003 (schema/test gaps)

- DS-05 benchmark: Mixed EAF + E&I + Steel (~80 objects)

- pmef-adapter-cadmatic: CADMATIC REST API adapter

- pmef-stress: CAESAR II .cii import/export adapter

- Full pmef-geom: AABB computation for all 15 primitives

- Full pmef-validate: jsonschema Rust crate integration

- 82/82 conformance tests passing

## v1.0 — Planned

- cargo test --workspace fully green

- Published crates on crates.io (pmef-core, pmef-io, pmef-cli)

- Published schemas on schema.pmef.org

- pmef.org website with full documentation

- IFC 4.3 export adapter (IfcPipeSegment, IfcFlowTerminal, IfcBeam)

- OpenUSD geometry layer: complete pmef: schema namespace + Omniverse connector

- First vendor Level 2 certification

- ISO/IEC JTC1 SC4 liaison statement

PMEF Specification v0.9.0-rc · https://github.com/pmef/specification · CC BY 4.0